1990 CSLAP Results:  Oquaga Lake

Parameter		Min	Avg	Max	Comments

pH			6.6	7.29	7.89	pH was somewhat variable, seasonally and annually, but still

within acceptable limits for most aquatic organisms

conductivity		56	61	79	conductivity was highly variable, but on average was

(µmho/cm)					comparable to readings in previous sampling seasons. The

conditions under which the sample with the maximum

reading was collected were likely not different from

collection conditions for other samples. Values are

indicative of softwater lakes

color			<1	2	5	water color was very low in Oquaga Lake- undetectable in

(platinum					two samples- and has been slowly decreasing since 1987.

color units)					The color in the lake is typical for lakes with very low levels

of dissolved organic matter

NO3			<0.02	<0.02	<0.02	nitrate concentrations have been at or below the lower

(mg/l)						analytical detection limit in every sample collected since

1987

total phosphorus		0.004	0.008	0.012	phosphorus concentrations have been fairly constant in the

(mg/l)						last four years, although concentrations vary somewhat

within a given sampling season. The measured values in

Oquaga Lake are indicative of unproductive lakes

chlorophyll a		0.63	2.06	3.01	algae levels (as measured by chlorophyll a) were low for the (µg/l)						entire summer, and did not vary enough to detect a

correlation with nutrient concentrations. Chlorophyll

readings were slightly lower than in the past two years

(much lower than in 1987), and are indicative of generally

unproductive lakes

Secchi disk		4.3	5.6	6.5	water transparency increased as the summer progressed,

transparency (m)					and was not dependent on changes in any other measured

parameters. Clarity was higher than in 1987 and 1989, and

comparable to readings in 1988, indicating that annual

variability is common. Values also continue to be indicative

of unproductive lakes, although clarity does not vary with

variation in nutrient concentrations, algae levels, or water

color

aquatic vegetation				no plant samples collected

general comments:

Water quality conditions, on average, have not changed very much at Oquaga Lake in the last four years. There appears to be some seasonal, and some annual, variability in a few of the measured parameters, including several parameters which are often linked to the common perceptions of water quality conditions (such as algae growth and water clarity), but this variability did not likely cause perceptions of degraded water quality. Moreover, the environment appears to be healthy in Oquaga Lake; nutrient levels remain very low, algae populations are sufficient to support most fisheries but not large enough to force impairments of recreational uses of the lake. Water transparency is higher than in most CSLAP lakes, and has remained so since the inception of CSLAP in 1987. It is unclear if this high water clarity has resulted in a proliferation of rooted aquatic vegetation growth, due to the lack of CSLAP vegetation data. The lake association should consider monitoring aquatic vegetation growth, to determine the extent and diversity of weed growth, and to detect the presence of non-native exotic plant species capable of dominating the aquatic plant community and affecting residents' use of the lake.

| Lakename | Date | Zbot | Zsd | TAir | TH2O | TotP | NO3 | TColor | pH | Cond25 | Chl.a |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Oquaga L | 7/14/90 | 30.0 | 4.250 |  |  | 0.011 | -0.02 | 5 | 7.23 | 64 | 3.01 |
| Oquaga L | 7/20/90 | 30.0 | 5.250 | 30 | 25 | 0.007 |  | 3 | 7.54 | 57 | 0.63 |
| Oquaga L | 8/3/90 | 30.0 | 5.250 | 27 | 24 | 0.008 | -0.02 | -1 | 7.89 | 56 | 2.08 |
| Oquaga L | 8/20/90 | 30.0 | 5.750 | 15 | 21 | 0.006 |  | 3 | 7.29 | 79 | 2.43 |
| Oquaga L | 9/1/90 | 30.0 | 6.250 | 25 | 23 | 0.004 | -0.02 | 2 | 6.60 | 57 | 1.34 |
| Oquaga L | 9/11/90 | 30.0 | 6.500 | 20 | 21 | 0.012 |  | -1 | 6.75 | 57 | 2.21 |
| Oquaga L | 9/27/90 | 30.0 | 6.250 | 21 | 13 | 0.008 | -0.02 | 3 | 7.74 | 57 | 2.75 |